Configuration of ELAN Card of MADM as Layer 2 switch

Due to shortage of FE ports in NIB switch, it was not possible to provide lease line like Internet LL or MPLS circuits at present. To overcome this problem in Guwahati, We configured the ELAN card of Tejas MADM as a Layer 2 switch.

Copper Pair

The diagram of connectivity as bellow

Modem

2 Pair to Ethernet  conv

Costumer

FE

CAT 5/6 Cable

MADM ELAN Card as L2  Switch

32/64 FE Port

FE

Link

STM

Station A

STM B

Station B

GE/FE Link

Costumer

Premises

Optical to

Electrical ETH Conv

OFC

NIB

P Router

FE

Following change or Steps for configure ELAN card as Layer 2 switch

We connect GE port 33 with NIB P-Router GE port.

STEP 1:  In switch configuration change Bridge Tag mode from QinQ to 802.1q switching

STEP 2:  Edit switch parameter of GE port 33, and given VLAN Id-1

STEP 3: Edit switch parameter of VCG 109, and given VLAN id-1

STEP 4: In VLAN provision a new VLAN:

VLAN Id-1000

VLAN Name-Prouter 1

Include GE port 33 and VCG 109 (both are tagged) and create the Vlan

STEP 5:  Click on QOS->Ports Rate Limiting and edit GE port 33

| Enable Ingress Rate Limiting |  |
| --- | --- |
| Ingress CIR (kbps) |  |
| Ingress CBS (KB) |  |
| Enable Ingress Pause Threshold |  |
| Pause Threshold |  |
| Resume Threshold |  |
| Enable Egress Rate Limiting |  |
| Egress CIR (kbps) |  |
| Egress CBS (KB) |  |

These steps are for initial configuration. Now for creation of every circuit steps are as follows,

Suppose we have to provide the 4 Mbps Ethernet circuits to any customer. We connect the customer link at FE port FE-1(electrical for ex), Vlan given by NIB is 3034.

STEP 1: Edit switch parameter of FE Port 1, and give VLAN ID-3034

STEP 2: In VLAN provision a new VLAN:

VLAN Id-3034

VLAN Name-Customer Name

Include GE port 33(Tagged), FE Port 1(UnTagged) and VCG 109 (tagged) and create the   Vlan

STEP 3: Click on QOS->Ports Rate Limiting and edit FE port 1

| Enable Ingress Rate Limiting |  |
| --- | --- |
| Ingress CIR (kbps) |  |
| Ingress CBS (KB) |  |
| Enable Ingress Pause Threshold |  |
| Pause Threshold |  |
| Resume Threshold |  |
| Enable Egress Rate Limiting |  |
| Egress CIR (kbps) |  |
| Egress CBS (KB) |  |

For Any Help Contact:

Vinay Kumar Shukla

JTO(Trans), Panbazar

Guwahati

Mob No-9435599402